Documentation -TFKISS- TheFirmware TNC Software Emulator

--------------------------------------------------------

was originally written by Mark Wahl, DL4YBG

This document was last changed on 11 July 1999

by Hans Mayer, OE1SMC and Berndt Josef Wulf, VK5ABN

0. Introduction

---------------

0.1. Licenses

-------------

TFKISS is based on TheFirmware by NORD><LINK. Refer to the copyright.txt

and alas.eng files included in this archive for more details on

licensing conditions.

Most of the AXIP API was taken from AX25IP source code written by

Mike Westerhof KA9WSB. His copyright notice is given below:

-------------------------------------------------------------------------

Copyright 1991, Michael Westerhof, Sun Microsystems, Inc.

This software may be freely used, distributed, or modified, providing

this footer is not removed.

-------------------------------------------------------------------------

The PID change code was written together with Joachim, DL8HBS.

The FLEXNET decoding code was written by Rene, DG0FT.

The remainder of the original source code was written by Mark Wahl

and is covered under the ALAS license.

0.2. Description

----------------

TNT, a hostmode terminal program, needs a TNC with TheFirmware connected

to a serial interface. There are some applications, where other

Configurations are needed:

- The TNC or PR-controller software is not compatible to TheFirmware

and its hostmode, but does support KISS mode

- The interface is not on a TNC, but a TCP/IP based network

(intranet/internet)

- TNT together with DPBOX shall be used directly connected to a

digipeater using KISS mode (hopefully with CRC) as interface

TFKISS was written to handle these applications. It is in fact a port

of the original TheFirmware by NORD><LINK, written for TNC2-hardware.

The low level interfaces were changed and adapted to unix-like operating

systems with support for  Linux, NetBSD and Solaris. On the radio

side a KISS and an AXIP interface have taken the place of the HDLC/modem

interface. The host connection is changed to a direct console interface (for

terminal mode) and a socket interface (for TNT).

1. Configuration

----------------

It is possible to operate TFKISS in several combinations. The radio side

can be a KISS interface, an AXIP interface or a combination of both.

On the host side a simple terminal mode can be used (nice for first tests)

or a socket interface to work together with TNT.

1.1. General configuration (tfkiss.ini)

---------------------------------------

This file is searched in the current directory and if not found in the

home directory of the user and in the directory defined by DEF_INI_DIR

during the configuration process.

Here is a description of parameters which can be defined in the main

configuration file.

1.1.1) General parameters

-------------------------

- kiss_active		Must be set to 1, if KISS interface shall be used,

otherwise set to 0.

- axip_active		Must be set to 1, if AXIP interface shall be used,

otherwise set to 0.

- tfkiss_conf_dir	Directory path where all tfkiss configuration

files are stored

- tfkiss_log_dir	Directory path where all tfkiss log files are

stored

- tfkiss_run_dir	Directory path where tfkiss status files are

stored

- tfkiss_procfile	Name of file containing the process id of the running

TFKISS process (for use with BOXSTART).

- tfkiss_errfile	Name for logfile to which logging data, warning and

error messages will be saved.

- tfkiss_parafile	Name of file where TFKISS will store internal para-

meters (callsign, txdelay, e.t.c.) at termination.

Only one of the following values is allowed:

- tfkiss_socket		Defines name of socket which is used as an interface

to TNT. The socket is defined by a valid pathname.

If the name does not start with a '/', 'tfkiss_dir'

is added at the beginning.

- tfkiss_extsocket	Defines name of socket which is used as an interface

to TNT. The socket can be an UNIX- or an IP-Socket.

Unix-sockets are defined by 'unix:<pathname>',

IP-Sockets by <IP-address/name>:<portnumber>.

Valid examples are:

unix:tfkiss

unix:/usr/local/dptnt/tfkiss/tfkiss

*:5000

localhost:5002

bla.blub.org:6000

If both tfkiss_socket and tfkiss_extsocket are left blank, TFKISS will start

in terminal mode on the console.

1.1.2) Parameters for KISS interface

------------------------------------

- device		Device used for a KISS interface, a serial device or

any other tty-device.

- tfkiss_lockfile	Lockfile to lock the KISS device against other

programs.

- speed			Baudrate used on KISS device

- fulldup_on_dama	Set to 1 for better DAMA performance, allows a quick

answer on the poll of the DAMA master independent of

DCD.

- kisstype		Type of KISS which shall be used on KISS device:

0: normal KISS without CRC

WARNING: This is HIGHLY insecure! You may lose

characters on the serial line WITHOUT knowing it.

Use this only for testing purposes!

1: SMACK, KISS with CRC16, developed by

Jan Schiefer, DL5UE

2: RMNC-KISS, KISS with CRC as implemented in

FLEXNET digipeater software

Note: There is NO autoprobing of the protocol

of the connected KISS device. You have to select

the correct mode manually!

c) Parameters for AXIP interface

- tfkiss_axipconfig	Name of file which will contain all parameters

concerning AXIP configuration.

Sample tfkiss.ini file:

-----------------------

# directory for config files

tfkiss_conf_dir /usr/local/share/tfkiss/conf

# directory for log files

tfkiss_log_dir /var/log

# directory for status files

tfkiss_run_dir /var/run

# file containing process id

tfkiss_procfile tfkiss.pid

# log file for errors

tfkiss_errfile tfkiss.err

# save file for parameters

tfkiss_parafile tfkiss.par

# UNIX-socket for connection to TNT

tfkiss_socket tfkiss_socket

# config-file for axip port

tfkiss_axipconfig tfkiss.cfg

# device for kiss port

device /dev/cua0

# lockfile for device used for kiss port

tfkiss_lockfile /usr/spool/uucp/LCK..cua0

# speed on device for kiss port

speed 19200

# switch kiss-tnc to fullduplex if DAMA detected

fulldup_on_dama 1

# IP-socket for connection to TNT

#tfkiss_extsocket *:5000

# type of KISS on kiss port: 0: KISS, 1: SMACK, 2: RMNC-KISS

kisstype 1

# enable (1) or disable (0) kiss port

kiss_active 1

# enable (1) or disable (0) axip port

axip_active 0

<EOF>

1.2. AXIP configuration (tfkiss.cfg)

------------------------------------

All AXIP configuration is defined in this file, which layout is similar

to that of the original AX25IP program:

- socket		Give mode of operation, several entries are

possible.

ip: AXIP according to RFC1226

udp: transport of AX25 data via an udp connection,

if no port is given, 10093 is used as a

default.

- mode			Defines the mode of operation of the AXIP part

of TFKISS.

tnc: data is given transparently to/from network

digi: data is digipeated using call defined as

mycall

- mycall		Digipeater callsign, only needed if mode digi is

used.

- loglevel		Logging function for debugging purposes, logging

data will be put to file defined by tfkiss_errfile.

- route			Defines routing data, gives network destination

and transport type for AX25 callsigns.

syntax: route <callsign> <hostname> [udp] [udp-port]

For default route <callsign> is substituted by

'default'.

Note: If default route is defined, no data will use

an available KISS interface!

sample tfkiss.cfg file:

# Sample tfkiss/ax25ip configuration file

#

# First select active socket types

# syntax: socket <ip/udp> [udp-port]

#

socket ip

socket udp

#

# Select the mode of operation. (digi or tnc)

#

mode tnc

#

# If you selected digi, you must define a callsign.  If you selected

# tnc mode, the callsign is optional

#

mycall nocall

#

# loglevel 0 - no output

# loglevel 1 - config info only

# loglevel 2 - major events and errors

# loglevel 3 - major events, errors, and AX25 frame trace

# loglevel 4 - all events

#

loglevel 0

#

# Define some routes.  One example routes all traffic for callsign db0zzz

# to a host named nexthost.bla.blub.  You can define as many as

# required.

# syntax: route <callsign> <hostname> [udp] [udp-port]

#

route db0zzz nexthost.bla.blub

route db0zzy-5 otherhost.bla.blub udp

route db0zzx-2 localhost udp 10094

#

# A catch-all is provided: this line sends all calls not specifically

# noted in the routing tables to otherhost.bla.blub.  Use this feature

# with great care -- the host on the other end may not appreciate all the

# traffic!

# syntax: route default <hostname> [udp] [udp-port]

#

#route default otherhost.bla.blub

#

<EOF>

2. Startup options

------------------

There are several options at startup of tfkiss that change the behaviour

of the program.

Usage : tfkiss [-i <init-file>] [-s <tfkiss-socket>] [-u] [-r] [-t]

-i	Defines an alternative init-file instead of the default tfkiss.ini.

This may be very useful when testing new system configurations.

-s	Defines an alternative socket for the TNT interface, even if

tfkiss_socket is not defined.

-u	Ignores an existing lockfile for the KISS device.

-r	Ignore and delete all system parameter stored in tfkiss_para file.

-t	Ingore the tfkiss_socket definition in tfkiss.ini and start

tfkiss on the console instead of the socket for TNT.

-x   This option switches a TNC2 from terminal-mode to kiss-mode.

It may be of particular interest to those operating TNC2 modems

with the "TheFirm" firmware, which will boot into terminal

terminal after powerup.

3. Finish TFKISS

----------------

There are 3 ways to exit TFKISS:

- By issuing the QRES command in terminal mode, which in

contrast to the original TNC-software terminates the program

and saves the tfkiss system parameter into the tfkiss_para

file.

- By sending a SIGTERM signal ( 15 ) to the process when using the

socket interface to TNT ('kill <tfkiss-process-ID>')

- sending a SIGUSR1 signal ( 16 )

this will switch back a tnc with thefirm from kiss

to a higher protocol, if started with -x option.

4. Extensions to TheFirmware

----------------------------

4.1. Parameter for txtail-length

--------------------------------

In KISS mode a value for the time between the last character put in the

HDLC controller and the switch off of the transmitter can be given.

This time is called 'txtail'.

It can be handled by the new extended TNC command '@TA'. The metric of

the value is 10ms, 4 means a txtail of 40ms.

4.2. Parameter for changing PID

-------------------------------

For special applications, like broadcast or TCP/IP different PID values

are needed. The PID can be changed using the new extended TNC command '@P'.

The argument for this command is the PID value in decimal. The PID will

be changed only on the currently selected channel.

4.3. Information about a changed PID

------------------------------------

If the remote station use a different PID than the default one 240 (0f0h),

a link status message will inform about this:

'PID changed to (EF)'

4.4. Decoding of frames in internal FLEXNET protocol

----------------------------------------------------

FLEXNET digipeaters use a special format for communication. Only one address

field (7 Bytes) is used to define source and destination of the data.

This specially encoded address field is displayed in readable form.

5. Contact

----------

Refer to the list below if you wish to get in contact with the current

maintainers of tfkiss:

Mailing List: tnt-devel@excelsior.kullen.rwth-aachen.de

Subscribe by sending an E-Mail to majordomo@excelsior.kullen.rwth-aachen.de

with "subscribe devel-tnt" in the main body of the message.

Hans Mayer (OE1SMC)

HAM-Radio: oe1smc@oe1xab.#oe1.aut.eu

E-Mail:    mayer@ping.at

URL:       http://www.bfl.at/mayer/ham

Matthias Hensler

E-Mail:    wsp@gmx.de

URL:       http://excelsior.kullen.rwth-aachen.de

Berndt Josef Wulf (VK5ABN)

HAM-Radio: vk5abn@vk5abn.#LMR.#SA.AUS.OC

E-Mail:    wulf@ping.net.au

73 - Berndt (VK5ABN)